# SARA 2026 — Speaker Notes

From Dish to Detection: Simplifying the Pulsar Observing Pipeline at DSES — Richard Hambly K0GD — Tue Aug 4, 4:15 PM EDT (2:15 PM MDT)

# Slide 1 — SOCIETY OF AMATEUR RADIO ASTRONOMERS  •  2026 CONFERENCE

Title. This is the story of a deliberate experiment — can we make pulsar observing something any motivated DSES member can do, not just our two experts?

Field session July 11, 2026 at the Haswell, Colorado 60-foot dish; results verified over the following two days.

# Slide 2 — FOR THE SARA PROGRAM

Skip live — or show for a beat while introducing the talk.

# Slide 3 — THE PROBLEM

Credit where due: this toolchain produces first-rate, professional results, and Rich and Dan built DSES's pulsar program on it. The point is not that the tools are bad — it's that the barrier to entry kept hands-on observing limited to two people.

Hundreds of pages of training material is a real number — this tool list is straight from the DSES pulsar training guide's system-configuration table.

The grayed row is the kicker: the professional world in common use runs even deeper — PSRCHIVE is a whole additional suite our two experts didn't even need.

# Slide 4 — THE ANSWER

Deliberate contrast with the previous slide: the wall of chips collapses to two.

On site, the “Spectrum Analyzer” does everything the observer needs; at home (or in the field), PRESTO v6 plus TEMPO2 does the analysis.

Polycos = "polynomial coefficients" — pre-computed cheat sheets for where a pulsar is in its rotation at any given moment.

The audience-ready version, two sentences: "To fold a recording you need to know the exact pulse phase at every sample — which depends on the pulsar's spin-down, the Earth's rotation and orbit, everything. Rather than solve the full timing model billions of times, TEMPO2 fits short polynomials — 'polycos' — that predict the phase over, say, an hour-long window each, and the folding code just evaluates those."

Slightly more depth if Q&A digs: a polyco file (polyco.dat) is generated for a specific pulsar, observatory, and time span from the pulsar's ephemeris (the .par file). Each block covers a time window with a handful of coefficients; prepfold evaluates the polynomial per sample to assign it a rotational phase bin. It's the classic trick that makes precise folding computationally trivial — dating back to the original TEMPO in the 1970s, which is why your slide mentions it: PRESTO v6 retired classic TEMPO, so the polycos now come from TEMPO2 instead — same concept, modern generator. Your pipeline does this invisibly: par file → TEMPO2 → polycos → fold. One more thing the observer never has to know about — which is, conveniently, the thesis of your talk.

PRESTO v6 dropped the classic-TEMPO dependency — barycentering is in-process and polycos come from TEMPO2 — and both installations are captured as re-runnable build scripts in the Society's repository (WSL on Windows, macOS).

Slide 5 — THE INSTRUMENT, LIVE

On site, the Spectrum Analyzer does everything the observer needs

# Slide 6 — THE INSTRUMENT

Everything the observer does on site — plan, survey, verify, record — in one package.

The analysis side (PRESTO) stays the professional standard; the acquisition side no longer requires it to be assembled by hand.

Distribution is from the Society's server with a built-in update checker.

# Slide 7 — THE EXPERIMENT — JULY 11, 2026

The Society's first live pulsar session using its own software.

Team of three; none are pulsar-processing experts in the sense of the previous slide.

Everything on site ran through the Spectrum Analyzer on one laptop driving the B210 at the 60-foot dish.

# Slide 8 — THE DATA

Same geometry for both recordings: 20 MHz at 420 MHz, 256 channels, 204.8 microsecond sampling, 32-bit — written directly as SIGPROC filterbank, the format PRESTO consumes.

Nearly 18 GB across the two targets.

# Slide 9 — RESULTS

Walk the figure: single clean pulse top-left,

single vertical track in phase-vs-time,

DM curve peaking near catalog.

The number's history: ~19.9-20.6 sigma across the original fold types as reported in the field; RFI mask + zap took it to 22.4; bridging a 0.558-second dropped-sample gap recovered full coherence at 28.0 sigma with DM back at 25.3. Chi-squared_red 16.75.

# Slide 10 — PEER REVIEW

The drift function was mapped directly:

22 consecutive 100-second fixed-period folds, each window's phase measured by cross-correlation — a smooth accelerating drip plus one discrete +0.219-rotation step at t ≈ 550 s.

The pad is 0.025% of the data, a copy of the preceding real noise — no synthetic signal; it only restores the time axis.

The 5.6-sigma jump is the coherence recovered when the two segments finally add in phase. Caveat: the padded file is for detection and profile work, not absolute timing. Full analysis in the refined fold PDFs beside the recordings.

# Slide 11 — RESULTS

Honesty is the credibility of the whole talk: the 2026-07-17 re-analysis applied the full B0329+54 cleanup — the significance stays ~9 sigma (noise-limited, not artifact-limited), but the best-fit parameters walk onto the catalog values for the first time. A predictive split-half coherence test came out consistent (~2 sigma) but not decisive — at 9 sigma total, subdividing runs out of signal. Processing gains are exhausted; the re-observation plan is on the roadmap: record after the timebase fix ships, 90+ minutes, multiple sessions for scintillation.

# Slide 12 — VERIFICATION

Two independent arguments.

Physics: the dispersion curve is the fingerprint — RFI doesn't have one.

Engineering: the same recordings re-analyzed on separately compiled PRESTO builds on two operating systems agree to three significant figures.

# Slide 13 — FIELD ENGINEERING

Crowd-pleaser story: the field box (Raspberry Pi 5, viewed over remote desktop) had 'broken' menus. Every symptom traced to one condition — headless operation yields a zero-by-zero screen. The fix shipped in v1.1.6 and the box was verified running it. Moral for any club running remote gear.

# Slide 14 — SHIPPED

Everything the field session asked for shipped within two days, and every machine — including the field box — converged on the release through the built-in updater. The line-endings epilogue is worth telling: the first Windows-built archive broke Unix launches; the field box exposed it within a day.

# Slide 15 — SHIPPED, AGAIN

The noise-floor scatter dropped ~34x (≈ sqrt of the averaging depth); max hold is now a true per-block peak detector, so a 51-microsecond burst reads at full amplitude instead of -27 dB. One release note worth mentioning: the default average is now linear power, so the displayed floor reads about +2.5 dB versus older versions — the old dB-domain average was biased low; the new number is the honest one. Verified live on the B210 at 1422 MHz with three weak narrowband test signals.

# Slide 16 — THE ROAD AHEAD

This is not a wish list — every item is specified in the repository's shared ROADMAP.md with design notes. The queue turns the remaining expertise into software: the planner replaces the Murmur/Stellarium planning step, one-click post-processing replaces the hand-run PRESTO pipeline for the common case, and timebase integrity fixes the defect peer review found. Science queue alongside: the B0950+08 campaign, and repeated B0329+54 sessions toward timing-grade data.

# Slide 17 — Three weeks later: the road-ahead list became a release

(no notes on this slide)

# Slide 18 — WHAT WE PROVED AT HASWELL

Close on the mission, not the software: this was about opening real radio astronomy to more members. Thanks: Rich and Dan built the program this stands on; the field team; and the report notes the role of AI-assisted engineering across three platforms. Questions.